| Process | CaM Kinase Assay |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Poisons: HEPES, EDTA, EGTA, Benzamidine, Sodium pyrophosphate, Dithiothreitol (DTT), Antipain, N-p-Tosyl-L-Lysine chloromethyl ketone (TLCK), Pepstatin A, Microcystin-LR (M-LR), Calcium chloride (CaCl2), Igepal, Sodium orthovanadate (Na3VO4), Aprotinin, Soybean trypsin inhibitor, Leupeptin, Magnesium acetate (MgA), Protein Kinase A Inhibitor (PKI), Protein Kinase C (PKC), Syntide 2, Calmodulin, Ethylene glycol. Poisons/Corrosives: Sodium fluoride (NaF), Phenylmethylsulfonyl fluoride (PMSF), Phosphoric acid. Radioactive: -32P-ATP. |
| Hazardous Equipment | Centrifuge. |
| Potential Hazards | HEPES, EDTA, EGTA, Benzamidine, Sodium pyrophosphate, DTT, Antipain, TLCK, Pepstatin A, M-LR CaCl2: irritating to eyes, skin, and respiratory system. Ethylene glycol: harmful if swallowed, irritating to eyes. Igepal: irritant, risk of serious damage to eyes. Soybean trypsin inhibitor, MgA, Calmodulin: Avoid contact and inhalation. Aprotinin, M-LR: harmful, may cause sensitization by inhalation and skin contact. Na3VO4, PMSF, Leupeptin, Phosphoric Acid, PKI, PKC, Calmodulin, M-LR, Syntide 2: Toxic, harmful by inhalation, contact with skin or eyes, and if swallowed. Leupeptin: possible risk to unborn child. PMSF, Phosphoric Acid: causes burns. PMSF, Phosphoric Acid, and NaF: contact with acids liberates very toxic gas. NaF: severe irritant. HEPES: hygroscopic. -32P-ATP: radioactive. |
| Personal Protective Equipment | Chemical safety glasses; nitrile or other appropriate chemical resistant gloves; full-buttoned front or back closing lab coat; closed toe shoes. Handling of reagents should be done using all personal protective equipment. |
| Engineering and Ventilation Controls | Handling of Phosphoric acid, NaF, PMSF, and Na3VO4 must be done in a chemical fume hood. |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, heat lamps or Bunsen burners; use metal tray or absorbent materials to contain any spilled solvent; never heat directly or work near a spark source; dispose of waste in properly labeled hazardous waste containers; be sure centrifuge and refrigerator can be used safely, that they don’t generate enough of a spark to ignite reagent. Ensure centrifuge closes correctly to avoid aerosolization. |
| Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other chemical spills should be reported to EH&S for clean-up. If radioactive materials are spilled, contact Prospective Health. Needed: absorbent materials brush/broom and dust pan, sealable plastic bags to contain clean up materials, protective clothing, gloves, and safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust; place clean up materials in plastic bag. For Phosphoric acid, cover spill with dry lime or soda ash, pick up, and place in a closed, properly labeled container. Wash spill site. Contact EH&S for removal of sealed container. Do not place clean up materials in regular waste. |
| #### Hazardous Waste Disposal | All waste material, including liquid and gel waste, will be discarded through the ECU hazardous waste management system. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |