Laboratory Safety Plan for :_Cell Culture

| # *Process | ## Feeding and maintenance of mammalian cells |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Flammable liquids-ethanol; Class 2 non-flammable-liquid nitrogen |
| *Hazardous Equipment | Bunsen burner |
| *Potential Hazards | Fire, freeze burns |
| *Personal Protective Equipment | Protective hood front; full-buttoned front or back closing lab coat; closed toed shoes. Use of insulated gloves when using liquid nitrogen. |
| *Engineering and Ventilation Controls | All handling, pipetting and use of ethanol is done in the isolation of the cell culture hood. |
| Special Use Procedures | Avoid ignition sources such as the Bunsen burner present in the hood. Avoid contact of exposed flesh with liquid nitrogen. |
| Special Handling and Storage Requirements | Wash hands frequently and upon removing personal protective equipment. |
| *Spill and Accident Procedures | Clean up spills only if you have the necessary materials on hand and are trained to do so. All other spills should be reported to EH&S for clean-up. Needed: absorbent materials (vermiculite, paper towels if you have nothing else on hand) brush and dust pan, plastic bags, sealable can to contain clean up materials, protective clothing, gloves, safety glasses, and mask to cover nose and mouth if needed. For minor spills: open windows or ventilate area; if spill occurs outside hood; cover liquid with absorbent material; brush into dust pan slowly and place into plastic bag; do not breathe dust from absorbent, place all clean up materials in plastic bag, then soap and water, discarding materials as above; contact EH&S for removal of sealed container, do not place clean up materials in regular waste. |
| *Waste Minimization Plan |  |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| #### Decontamination Procedures | Not applicable |
| Animal Care Precautions | Not applicable |
| *Chemical Procurement | Check Recy-chem before purchasing. Obtain the minimum necessary for the procedure. |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)