Laboratory Safety Plan for __Column Chromatography__________________

| # *Process | ## Column Chromatography |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Sodium azide-poison, explosive under high heat. |
| *Hazardous Equipment | Glass columns, electrical equipment |
| *Potential Hazards | Broken glass; inhalation or absorption of poison into body; liquid spills could cause electrical shock. Sodium azide - Poison! May be fatal if swallowed or absorbed through skin. Harmful if inhaled. Hazardous solid. Contact with other material may cause fire or explosion. Causes irritation to skin, eyes and respiratory tract. Affects central nervous system, kidneys, and cardiovascular system. |
| *Personal Protective Equipment | Rubber gloves, full-buttoned lab coat, close toed shoes, safety glasses. If splashing or aerosolization of chemicals/samples may be an issue, use safety goggles. |
| *Engineering and Ventilation Controls | Weigh sodium azide out under hood, use extreme caution to prevent dust. |
| Special Use Procedures | N/A |
| Special Handling and Storage Requirements | Avoid open flame or sources of high heat (hot plates, heat lamps), store away from heat. |
| *Spill and Accident Procedures | For minor spills: Remove all sources of ignition, use absorbant materials (vermiculite, paper towels if nothing else available), clean up in a manner that does not disperse dust into the air, place all clean up materials in container for disposal through EH&S. |
| *Waste Minimization Plan | Review procedures to use the smallest amount of hazardous chemicals possible. |
| #### *Hazardous Waste Disposal | Waste material will be discarded through the ECU hazardous waste management system |
| #### Decontamination Procedures | N/A |
| Animal Care Precautions | N/A |
| *Chemical Procurement |  |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |