Laboratory Safety Plan for Determination of Enzymatic Activity of Peas

| # *Process | ## Determination of Enzymatic Activity of Peas |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Caustic-ferric chloride, hydrochloric acid, trichloroacetic acid, magnesium sulfate, hydroxylamine hydrochloride. Toxic/poison-ferric chloride, trichloroacetic acid, magnesium sulfate, hexadecyltrimethylammonium bromide(CTAB), hydroxylamine hydrochloride |
| *Hazardous Equipment | Spectrophotometer |
| *Potential Hazards | Chemical burns; inhalation and absorbtion of toxic chemicals into body, lung damage from inhalation of CTAB |
| *Personal Protective Equipment | Gloves, fully buttoned lab coat, closed toed shoes, safety glasses |
| *Engineering and Ventilation Controls | Use CTAB and trichloroacetic acid in chemical fume hood |
| Special Use Procedures | None |
| Special Handling and Storage Requirements | Dispose of reaction materials in the indicated container to be picked up by E&S |
| *Spill and Accident Procedures | For minor spills: Open windows or ventilate lab; use absorbant materials such as vermiculite (or paper towels if nothing else is available) to clean up magnesium sulfate, ferric chloride and CTAB and hydroxylamine hydrochloride. Sweep slowly, without raising dust, place into labeled container for disposal. Cover trichloroacetic acid with excess sodium bicarbonate, sweep without raising dust and place into labeled container for disposal. For major spills, notify EH&S for clean-up. |
| *Waste Minimization Plan | None |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system |
| #### Decontamination Procedures | N/A |
| Animal Care Precautions | N/A |
| *Chemical Procurement |  |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |