SAMPLE: Laboratory Safety Plan for Formaldehyde Use

| # Process | N/A |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Formaldehyde |
| # Hazardous Equipment | N/A |
| # Potential Hazards | Inhalation of toxic vapors; absorption through the skin, eyes, and transport of toxic substance dissolved in them into the body; tissue damage. Formaldehyde is corrosive and may cause serious burns; irritant contact to the skin will cause inflammatory effect; will cause one to become sensitized; classified as possible cancer causing agent;. |
| Personal Protective Equipment | Chemical splash goggles; nitrile or neoprene gloves; full buttoned front or back closing lab coat; closed toe shoes |
| Engineering and Ventilation Controls | All dilution and handling of chemical should be done under chemical fume hood. |
| Special Use Procedures | N/A |
| Special Handling and Storage Requirements | Solutions prepared in chemical fume hood. Hands and working surface washed following use. Stored in a secure, closed container below shoulder level in secondary containment with Class 9 carcinogens. Use only in designated areas. (Identify for your lab.) |
| # Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other spills should be reported to EH&S for clean-up. Needed: absorbent materials, brush and dust pan, plastic bags, seal able can to contain clean up materials, protective clothing, gloves, safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust from absorbent. Contact EH&S for removal of sealed waste clean-up material. Do not place clean up materials in regular waste. |
| # Hazardous Waste Disposal | All waste material, including liquid waste, will be discarded through the ECU hazardous waste management system. Samples for discard will be separated from liquid formaldehyde and discarded as red bag biological waste. Fixed slides may be discarded in a sharps container or broken glass container. |
| Decontamination Procedures | Not applicable. |
| Animal Care Precaution | Not applicable. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |