Laboratory Safety Plan for: Glomerular Filtration Test (used clinically)

| # *Process | ## Renal clearance of Glofil-125 (Sodium Iothalamate I-125) in patients. Inject 25 Ci in each patient. |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Radioactive Chemical |
| *Hazardous Equipment | Centrifuge, Needles |
| *Potential Hazards | Patient and user exposure to radiation. Contamination through inhalation, skin contact, open cuts, eyes, ingestion. May result in increased thyroid radiation dose if inhaled or ingested. Slight corrosive effect and contamination with I-125 if not washed immediately from skin. Causes eye irritation. May cause allergic reactions in people allergic to iodine or iodinated substances. |
| *Personal Protective Equipment | Chemical splash goggles, latex gloves, lab coat. Lead shield placed around sharps container. A ½ inch plexi-glass is placed between the user and product. |
| *Engineering and Ventilation Controls | The Glofil-125 is drawn in tuberculin syringe in the fume hood with a plexi-glass between the user and the product. |
| Designated Use Area for Carcinogens, Reproductive Toxins or Acute Toxins | There are two separate rooms within Brody 3W27, which are used for preparing syringes containing the product and the other is used to administer injection to the patient. The remaining lab is used for patient waiting. |
| Special Use Procedures | Glofil-125 is administered by a subcutaneous injection on back of the upper arm of the patient. |
| Special Handling and Storage Requirements | Product is stored at 4 C in a lead shield inside a plexi-glass container. |
| *Spill and Accident Procedures | Any urine or blood specimen spilled is absorbed immediately by absorbent paper covering all surfaces. For small spills of the glofil-125, the product is taken up with sand, earth or other non-combustible absorbent material. For large spills, Radiation Safety is called immediately and all personnel are kept from the area. |
| *Waste Minimization Plan | Difficult to minimize waste since urine output depends on the amount of fluid the patient drinks. |
| #### *Hazardous Waste Disposal | All hazardous waste is picked up and discarded by Radiation Safety Office. |
| #### Decontamination Procedures | All surface areas, glassware and restroom are decontaminated with Lift-Away super decontaminant. |
| Animal Care Precautions | Not applicable. |
| *Chemical Procurement | Only 1 vial is kept in the lab at all times. Ordered when needed. |
| *Revision Date | Insert of Date: 3-4-2002 |

Lab location:

Investigator:

Primary User: