# SAMPLE: Laboratory Safety Plan for Gram Stain Procedure

| # Process | Gram Stain Procedure |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Irritant- crystal violet, methylene blue, potassium iodide, ethanol; carcinogenic- crystal violet; hepatotoxin- ethanol; toxic- crystal violet |
| Hazardous Equipment | Bunsen Burner (or other heat/flame source); autoclave; glass slides |
| Potential Hazards | Fire; tissue damage; possible carcinogenic effects from crystal violet; possible liver damage from ethanol; burn potential from Bunsen burner; potential burn from autoclave; possible noxious fumes generated from autoclave; broken glass |
| Personal Protective Equipment | Chemical safety glasses; neoprene gloves; heat resistant gloves when using autoclave; full-buttoned front or back closing lab coat; closed toed shoes |
| Engineering and Ventilation Controls | Turn on extraction fan when using autoclave; if using BioSafety Level 2 or higher, use an appropriate BioSafety Cabinet |
| Special Use Procedures | Keep long hair pulled back behind your head; secure any loose clothing; only have burner on when you are actively using it; turn burner off when not in use; be aware of any vapors possibly released from using autoclave |
| Special Handling and Storage Requirements | Know biosafety risk groups; handle all cultures as potential pathogens; all cultures should be properly identified such that the degree of risk can be ascertained in the event if an accident |
| Spill and Accident Procedures | Laboratory supervisor should be notified; spill should be flooded with disinfectant and covered with paper towels and let sit for 20 minutes; mop up spill and place paper towels in bag for autoclaving; wipe over area again with paper towels moistened with disinfectant; if a major spill, room may have to be evacuated until properly cleaned by spill clean up team; contaminated broken glass should be soaked in disinfectant solution for 20 minutes, swept up and autoclaved, and put in broken glass container for disposal; uncontaminated glass should be put in broken glass container for disposal |
| Hazardous Waste Disposal | Contaminated waste should be placed in biohazard bag for disposal by Prospective Health; any contaminated waste that is autoclaved is ready for disposal in a sanitary landfill |
| Decontamination Procedures | Laboratory bench tops must be cleaned with disinfectant before and after procedure; all material that can be reused should be washed with disinfectant before being autoclaved; biosafety cabinet should be wiped down with disinfectant starting from front to back and then from the top to the bottom |
| Animal Care Precautions | Not applicable. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |