Laboratory Safety Plan for ___Immunoblot analyses

| # *Process | ## Immunoblot analyses |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Flammable liquids-alcohols, carcinogens-polyacrylamide |
| *Hazardous Equipment | Gel electrophoresis power supply and heating plate |
| *Potential Hazards | Fire; inhalation of toxic vapors, absorption through skin, electrical shock, direct contact with heating plate |
| *Personal Protective Equipment | Safety glasses/mask; nitrile gloves, lab coat, close toed shoes |
| *Engineering and Ventilation Controls | See designated area |
| Designated Carcinogen, Reproductive Toxin or Acute Toxin Use Area | Polyacrylamide will be procured as premixed liquid or mixed under the chemical fume hood. |
| Special Use Procedures | When using electrophoresis apparatus: check the physical integrity of equipment; follow equipment operating instructions carefully; use physical barriers to prevent inadvertent contact with apparatus; only use insulated lead containers; turn off power before connecting electrical leads; use away from water and water sources; use electrical interlocks; turn power off before opening lid or reaching into container; do not disable safety devices; connect one lead at a time using only one hand, making sure hand is dry; use warning signs to alert others of potential electrical hazard |
| Special Handling and Storage Requirements | Avoid all potential sources of ignition (Bunsen burners, hot plates); use absorbent materials to contain any spilled solvent; dispose of waste or used ethanol in labeled hazardous waste containers. |
| *Spill and Accident Procedures | Trained personnel are to clean up spills of hazardous chemicals only if the necessary materials are available. All other spills are to be reported to EH&S for clean-up. |
| *Waste Minimization Plan | Purchase 1 semester supply |
| #### *Hazardous Waste Disposal | All chemical waste material will be discarded through the ECU hazardous waste management system by notifying the EH&S office |
| #### Decontamination Procedures | Not applicable. |
| Animal Care Precautions | Not applicable. |
| *Chemical Procurement | Quantity required and means of obtaining the minimum necessary. |
| *Revision Date | Insert ____ of ____ Lab |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)