| Process | MTT Proliferation Assay |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Combustible Liquid/ Poison: DMSO; Poison: MTT |
| Hazardous Equipment | Centrifuge. |
| Potential Hazards | DMSO: Irritating to eyes, respiratory system, and skin; readily absorbed through skin; combustible liquid; moisture sensitive. MTT: Avoid contact and inhalation; hygroscopic. |
| Personal Protective Equipment | Chemical safety glasses; Neoprene or natural rubber gloves are best for DMSO; full buttoned front or back closing lab coat; closed toe shoes. |
| Engineering and Ventilation Controls | All handling of these reagents should be done while wearing all personal protective equipment. |
| Special Handling and Storage Requirements | MTT: store at 2-8C, and protect from light. Avoid ignition sources such as hot plates, heat lamps or Bunsen burners; use metal tray or absorbent materials to contain any spilled solvent; never heat directly or work near a spark source; be sure centrifuge and refrigerator can be used safely, and that they don’t generate enough of a spark to ignite reagent. |
| Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other spills should be reported to EH&S for clean-up. Needed: absorbent materials, brush and dust pan, sealable plastic bags to contain clean up materials, protective clothing, gloves, safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust from absorbent; place clean up materials in plastic bag. Absorb spill with sand or vermiculite and place in properly labeled hazardous waste container. Wash spill area. Contact EH&S for removal of sealed container. Do not place clean up materials in regular waste. |
| #### Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |
| Revision Date: |  |