Laboratory Safety Plan for Non-Cyclic Photosynthetic Electron Transport

| # *Process | ## Non-Cyclic Photosynthetic Electron Transport |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Poison: potassium hydroxide; Corrosive: potassium hydroxide; Flammable: sodium hydrosulfite; Water reactive: sodium hydrosulfite |
| *Hazardous Equipment | Lamps |
| *Potential Hazards | Fire; contact burns from lamps; chemical burns from potassium hydroxide, inhalation and absorption of poison; heat reaction of sodium hydrosulfite contact with moisture. Sodium hydrosulfite is a flammable solid which may ignite with moisture and air. |
| *Personal Protective Equipment | Nitrile gloves, fully-buttoned lab coat, safety glasses, close-toed shoes. |
| *Engineering and Ventilation Controls | Use sodium hydrosulfite in hood. |
| Special Use Procedures | Containers of sodium sulfite may be hazardous when empty since they retain product residues (dust, solids). Remove all ignition sources. |
| Special Handling and Storage Requirements | Keep sodium hydrosulfite away from heat sources and moisture, check lamps for frayed cords, dispose of waste through EH&S |
| *Spill and Accident Procedures | For minor spills: use absorbent material such as vermiculite or paper towels to soak up liquid spills, sweep up in a manner that does not raise dust, dispose of all cleanup materials through EH&S. DO NOT apply water or any moisture to sodium hydrosulfite spills. |
| *Waste Minimization Plan | Evaluate the substitution of less hazardous chemicals and use the smallest amount possible. |
| #### *Hazardous Waste Disposal | Dispose of all waste materials through EH&S |
| #### Decontamination Procedures | N/A |
| Animal Care Precautions | N/A |
| *Chemical Procurement | 100 g of potassium hydroxide and 5 g of sodium hydrosulfite are sufficient for one year |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |