Laboratory Safety Plan for ___Polysome isolation/recovery/ribonuclease protection assay_

| # *Process | ## Polysome isolation/recovery/ribonuclease protection assay |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Flammable liquids-alcohols, formamide, ether, carcinogens-chloroform, polyacrylamide, corrosive-phenol |
| *Hazardous Equipment | Ultracentrifuge and heating plate |
| *Potential Hazards | Fire; inhalation of toxic vapors, absorption through skin, eyes, and transport of toxic substance dissolved in them into the body: chloroform/formamide vapors are toxic and damaging to tissues, absorption through skin; phenol is corrosive and direct contact can cause serious burns, vapor is also corrosive to tissue; direct contact with the heating plate will burn tissue. |
| *Personal Protective Equipment | Safety glasses; nitrile gloves, lab coat, close toed shoes |
| *Engineering and Ventilation Controls | All handling, pipetting, dilutions of hazardous chemicals must be done in the chemical fume hood located in ______; containers for these materials can only be removed from hood for centrifuging if securely capped |
| Designated Carcinogen, Reproductive Toxin or Acute Toxin Use Area | Toxic substances will be measured and mixed in the chemical fume hood. |
| Special Use Procedures | Double glove when handling toxics. Wash hands upon removing gloves. |
| Special Handling and Storage Requirements | Avoid all potential sources of ignition (Bunsen burners, hot plates); use absorbent materials to contain any spilled solvent; dispose of waste or used ethanol, phenol, formamide and chloroform in labeled hazardous waste containers. |
| *Spill and Accident Procedures | Trained personnel are to clean up spills of hazardous chemicals only if the necessary materials are available. All other spills are to be reported to EH&S for clean-up. |
| *Waste Minimization Plan | Minimum quantity will be purchased for 1 semester work. |
| #### *Hazardous Waste Disposal | All chemical waste material will be discarded through the ECU hazardous waste management system by notifying the EH&S office. Biomedical waste is disposed by placing this material in the biomedical waste container located in __________. |
| #### Decontamination Procedures | Not applicable. |
| Animal Care Precautions | Not applicable. |
| *Chemical Procurement | Quantity required and means of obtaining the minimum necessary. |
| *Revision Date | Insert ____ of ____ Lab: |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)