| Process | RIPA Protein Cell Lysis |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Poisons: Tris, Sodium Chloride (NaCl), Igepal, Sodium Dodecyl Sulfate (SDS), Deoxycholic Acid (DOC), Aprotinin, Leupeptin, Sodium Orthovanadate (Na3VO4), Glycerol, Bromphenol Blue (BPB). Poison/Corrosive: Phenylmethylsulfonyl fluoride (PMSF). Poison/Combustible Liquid: 2-Mercaptoethanol (2-Me). |
| Hazardous Equipment | Centrifuge. |
| Potential Hazards | Tris, SDS, 2-Me: causes irritation to skin, eyes, and respiratory tract. 2-Me: combustible liquid, moisture sensitive. DOC, NaCl: harmful if swallowed, irritating to respiratory tract. Igepal, NaCl: causes eye irritation and damage. Aprotinin: sensitization by inhalation and skin contact. Leupeptin: risk to unborn child. Leupeptin, Na3VO4, Glycerol (hygroscopic), BPB, PMSF: harmful by inhalation, skin contact, and ingestion. PMSF: causes burns, contact with acids liberates very toxic gas, moisture sensitive. |
| Personal Protective Equipment | Chemical safety glasses; nitrile gloves; full-buttoned front or back closing lab coat; closed toe shoes. |
| Engineering and Ventilation Controls | Handle all reagents using all personal protective equipment. Use Na3VO4, PMSF, and 2-Me in chemical fume hood only. |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates or Bunsen burners; never heat directly or work near a spark source; be sure centrifuge and refrigerator can be used safely and don’t generate enough of a spark to ignite reagent. |
| Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other spills should be reported to EH&S for clean-up. Needed: absorbent materials brush/broom and dust pan, sealable plastic bags to contain clean up materials, protective clothing, gloves, safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust; place clean up materials in plastic bag. Contact EH&S for removal of sealed container. Do not place clean up materials in regular waste. |
| #### Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount. |
| Revision Date: |  |