| Process | Polyacrylamide Gel Electrophoresis with Sodium Dodecyl Sulfate Preparation (SDS-PAGE) |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Poison: Tris, Sodium dodecyl sulfate (SDS), Glycine. Poison/Carcinogen: ProtoGel (acrylamide/bisacrylamide). Poison/Corrosive/Flammable Liquid: TEMED. Poison/Corrosive: hydrochloric acid. Poison/Oxidizer/Flammable Solid: Ammonium Persulfate (APS). |
| Hazardous Equipment | Electrophoresis apparatus. |
| Potential Hazards | Tris, SDS, APS: irritating to eyes, skin, and respiratory tract. Glycine: avoid contact and inhalation. ProtoGel: toxic in contact with skin and is swallowed, may cause cancer, acrylamide and bisacrylamide may cause CNS damage. TEMED: flammable liquid, may cause irreversible eye damage, causes burns, harmful if inhaled or swallowed. APS: oxidizing, contact with combustible material may cause fire, may cause sensitization by inhalation and skin contact. Potential electrical shock from electrophoresis apparatus because of high voltage and conductive fluid. |
| Personal Protective Equipment | Chemical safety glasses; nitrile gloves; full buttoned front or back closing lab coat; closed toe shoes. |
| Engineering and Ventilation Controls | All personal protective equipment should be used when handling powder reagents. HCl and APS should only be used in a chemical fume hood. |
| Special Use Procedures | When using electrophoresis apparatus: check the physical integrity of equipment; follow equipment operating instructions carefully; use physical barriers to prevent inadvertent contact with apparatus; only use insulated lead containers; turn off power before connecting electrical leads; use away from water and water sources; use electrical interlocks; turn power off before opening lid or reaching into container; do not disable safety devices; connect one lead at a time using only one hand, making sure hand is dry; use warning signs to alert others of potential electrical hazard. |
| Special Handling and Storage Requirements | Avoid ignition sources; use absorbent materials to contain any spill; dispose of waste in properly labeled hazardous waste containers; be sure electrophoresis apparatus can be used safely. |
| Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other spills should be reported to EH&S for clean-up. Needed: absorbent materials, brush and dust pan, sealable plastic bags to contain clean up materials, protective clothing, gloves, safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust. For HCl, cover spill with dry lime, sand, or soda ash; place into properly labeled disposal container using non-sparking tools. For TEMED, cover spill with sand, diatomite, acid binders, universal binders, or sawdust; use a neutralizing agent; collect into properly labeled disposal container. For APS, cover spill with sand or vermiculite; place into properly labeled disposal container; place clean up materials in plastic bag and discard as above. Contact EH&S for removal of sealed container. Do not place clean up materials in regular waste. |
| #### Hazardous Waste Disposal | All waste material, including liquid and gel waste, will be discarded through the ECU hazardous waste management system. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |
| Revision Date: |  |