Laboratory Safety Plan for-Specimen-Preservation

| # *Process | ## Specimen preservation and examination |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Flammable liquids – Ethanol Poisonous liquids – Formaldehyde |
| *Hazardous Equipment | none |
| *Potential Hazards | Fire; inhalation of toxic vapors, absorption through skin, eyes, and transport of toxic substance dissolved in them into the body; poisons can increase the hazard in any fire due to inhalation of volatilized reagent. |
| *Personal Protective Equipment | Safety glasses and face shield; nitrile gloves (resistant to these materials) full-buttoned front or back closing lab coat; close toed shoes. |
| *Engineering and Ventilation Controls | Solutions will be prepared and manipulated in the chemical fume hood. |
| Special Use Procedures |  |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, heat lamps or Bunsen burners; use metal tray or absorbent materials to contain any spilled solvent; never heat directly or work near a spark source; dispose of waste or used ethanol, and formaldehyde in labeled hazardous waste containers. |
| *Spill and Accident Procedures | Clean up spills only if you have the necessary materials on hand and are trained to do so. All other spills should be reported to EH&S for clean-up. Needed: absorbent materials (vermiculite, paper towels if you have nothing else on hand) brush and dust pan, plastic bags, sealable can to contain clean up materials, protective clothing, gloves, safety glasses, and mask to cover nose and mouth if needed. For minor spills: open windows or ventilate area; if spill occurs outside hood; cover liquid with absorbent material; brush into dust pan slowly and place into plastic bag; do not breathe dust from absorbent, place all clean up materials in plastic bag, then soap and water, discarding materials as above; contact EH&S for removal of sealed container, do not place clean up materials in regular waste. |
| *Waste Minimization Plan | Every effort will be made to minimize waste. |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| #### Decontamination Procedures | Not applicable |
| Animal Care Precautions | Not applicable |
| *Chemical Procurement | Quantity required and means of obtaining the minimum necessary. |
| *Revision Date | 6 March 2002 |