| Process | Sulforhodamine B Viability Assay |
| --- | --- |
| Hazardous Chemical/ Chemical Class | Poison/Corrosive: Trichloroacetic Acid (TCA). Poisons: Sulforhodamine B (SRB), Tris. Explosive/Flammable Liquid/Poison/Corrosive: Acetic Acid |
| Hazardous Equipment | Centrifuge. |
| Potential Hazards | TCA: causes severe burns to contact area; harmful if swallowed or inhaled, inhalation may cause lung damage, hygroscopic. SRB: avoid contact and inhalation. Acetic Acid: causes severe burns; harmful in contact with skin; combustible; flammable, keep away from sources of ignition. Tris: irritating to eyes, skin, and respiratory tract. |
| Personal Protective Equipment | Chemical safety glasses; nitrile gloves; full-buttoned front or back closing lab coat; closed toe shoes; facial dust mask. |
| Engineering and Ventilation Controls | Wear all personal protective equipment when handling powder reagents. Use Acetic acid in fume hood only. |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, heat lamps or Bunsen burners; use metal tray or absorbent materials to contain any spilled solvent; never heat directly or work near a spark source; dispose of waste in properly labeled hazardous waste containers; be sure centrifuge and refrigerator can be used safely, that they don’t generate enough of a spark to ignite reagent. |
| Spill and Accident Procedures | Clean spills only if proper materials are available and if researcher is properly trained to do so; all other spills should be reported to EH&S for clean-up. Needed: absorbent materials, brush/broom and dust pan, sealable plastic bags to contain clean up materials, protective clothing, gloves, safety glasses. For minor spills: ventilate area; if spill occurs outside hood, cover liquid with absorbent material; slowly brush into dust pan and place in plastic bag; do not breath the dust; place clean up materials in plastic bag and discard as above. For TCA, cover spill with excess sodium bicarbonate and carefully sweep up material and place in properly labeled, corrosion resistant container; if material is mixed with water, neutralize with sodium bicarbonate, cover with vermiculite, dry sand, or earth, and place in properly labeled corrosive resistant container. For Acetic acid, cover spill with dry lime or soda ash, place in properly labeled disposal container; wash spill site. Contact EH&S for removal of sealed container. Do not place clean up materials in regular waste. |
| #### Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| Chemical Procurement | Quantity required and means of obtaining the minimum amount necessary. |
| Revision Date: |  |