Laboratory Safety Plan for:_Weighing out chemicals

| # *Process | ## Weighing out chemicals |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | All classes |
| *Hazardous Equipment | Not applicable |
| *Potential Hazards | Exposure, inhalation of toxic vapors or fine particles, absorption through skin, eyes, and transport of toxic substance dissolved in them into the body. |
| *Personal Protective Equipment | HEPA filtered respirator or N100 mask for toxic powders, N95 for non-toxic dusts and allergens, safety glasses; nitrile gloves (resistant to these materials) full-buttoned front or back closing lab coat; close toed shoes. |
| *Engineering and Ventilation Controls | Where possible weigh materials under fume hood. Mix solutions under chemical fume hood. |
| Special Use Procedures | Respiratory protection must be fit tested by EH&S on an annual basis. |
| Special Handling and Storage Requirements | Gloves changed frequently, never taken out of the lab. Wash hands upon removal of gloves. Wash face upon removal of respirator. |
| *Spill and Accident Procedures | Clean up spills only if you have the necessary materials on hand and are trained to do so. All other spills should be reported to EH&S for clean up. Needed: absorbent materials (vermiculite, paper towels if you have nothing else on hand) brush and dust pan, plastic bags, sealable can to contain clean up materials, protective clothing, gloves, safety glasses, and mask to cover nose and mouth if needed. For minor spills: open windows or ventilate area; if spill occurs outside hood; cover liquid with absorbent material; brush into dust pan slowly and place into plastic bag; do not breathe dust from absorbent, place all clean up materials in plastic bag, then soap and water, discarding materials as above; contact EH&S for removal of sealed container, do not place clean up materials in regular waste. |
| *Waste Minimization Plan | Use only as much as needed. |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| #### Decontamination Procedures | Clean the balance stage and pan after weighing is complete. Soap and water, water or 70% alcohol (acute toxins). |
| Animal Care Precautions | Not applicable |
| *Chemical Procurement | Quantity required and means of obtaining the minimum necessary. |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)