Laboratory Safety Plan for :  Western blots

| # *Process | ## Gel elecxtrophoresis and blotting of proteins to membranes |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Flammable liquids-methanol; poisons – menthanol, amido black, lauryl sulfate, Tris-glycine gels (2.6% acrylamide) |
| *Hazardous Equipment | Power supplies, X-ray film developer |
| *Potential Hazards | Electrical shock, fire, inhalation of toxic vapors, absorption through skin, eyes, and transport of toxic substance dissolved in them into the body; poisons can increase the hazard in any fire due to inhalation of volatilized reagent. |
| *Personal Protective Equipment | Safety goggles; nitrile gloves (resistant to these materials) full-buttoned front or back closing lab coat; close toed shoes. |
| *Engineering and Ventilation Controls | All methanol must be measured in the chemical fume hood. Dry acrylamide must be weighed and mixed under the fume hood. |
| Special Use Procedures |  |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, heat lamps or Bunsen burners; use polystyrene tray or absorbent materials to contain any spilled solvent; never heat directly or work near a spark source; dispose of waste or used methanol in labeled hazardous waste containers; be sure that centrifuge and refrigerator can be used safety - that they don't generate enough of a spark to ignite reagent. Power supplies are kept above the level of solvents and solutions |
| *Spill and Accident Procedures | Clean up spills only if you have the necessary materials on hand and are trained to do so. All other spills should be reported to EH&S for clean-up. |
| *Waste Minimization Plan | Utilize pre-mixed acrylamide gel. Completely dry gels for disposal. |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| #### Decontamination Procedures | Not applicable |
| Animal Care Precautions | Not applicable |
| *Chemical Procurement | Quantity required and means of obtaining the minimum necessary. Order Tris-Glycine gels just before use. |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)