Laboratory Safety Plan for: Micro-scale batch reactor – acylation with acetic anhydride or glacial acetic acid

| # *Process | ## Acylation reactions with acetic anhydride / micro-scale batch reactor |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Combustible liquids – acetic anhydride, alcohols, glacial acetic acid. Corrosive liquids – glacial acetic acid. Water reactive liquids – acetic anhydride. Toxic substances – salicylic acid, acetylsalicylic acid. |
| *Hazardous Equipment | None. |
| *Potential Hazards | Fire; Inhalation of toxic or corrosive vapors; Absorption through skin, eyes, and transport of toxic substances dissolved in them into the body; Poisons can increase the hazard in fire due to inhalation of volatilized reagent. Acetic anhydride reacts with water to produce a significant amount of heat and acetic acid. |
| *Personal Protective Equipment | Safety glasses; Latex or nitrile gloves; Lab coat; Close toed shoes. |
| *Engineering and Ventilation Controls | Handling, pipetting, and dilutions of reagents must be done in a chemical fume hood; Containers of these materials should be removed from the fume hood only when tightly capped. |
| Designated Carcinogen, Reproductive Toxin or Acute Toxin Use Area | Toxic substances will be used in the chemical fume hood. |
| Special Use Procedures | Double glove when handling toxics. Wash hands upon removing gloves |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, heat lamp, or Bunsen burners; use absorbent materials to contain spilled materials. Never heat directly or work near a spark source. Dispose of waste in labeled hazardous waste containers. Acetic anhydride reacts with water to produce heat and acetic acid. Contact with large quantities of water should be avoided. |
| *Spill and Accident Procedures | Clean up spills only if you have the necessary materials on hand and are trained to do so. All other spills should be reported to EH&S for clean-up. Needed: absorbent materials (vermiculite, paper towels if you have nothing else on hand) brush and dust pan, plastic bags, sealable can to contain clean-up materials, protective clothing, gloves, safety glasses, and mask to cover notes and mouth if needed. For minor spills: open windows or ventilate area; If spill occurs outside hood; cove liquid with absorbent material; brush into dust pan slowly and place into plastic bag; do not breathe dust from absorbent, place all clean up materials in plastic bag, then wash with soap and water, discarding materials as above; contact EH&S for removal of sealed container, do no place clean up materials in regular waste. |
| *Waste Minimization Plan | Minimize the amount of solvents used in batch reactions. Recover waste solvent through distillation where possible. |
| #### *Hazardous Waste Disposal | All waste material will be discarded though the ECU hazardous waste management system. |
| #### Decontamination Procedures | Not applicable (applies to biosafety cabinets or other areas that require specific clean up procedures after a process). |
| Animal Care Precautions | Not applicable |
| *Chemical Procurement | Check recy-chem prior to purchase. Obtain the minimum quantities needed |
| *Revision Date | 02/09/2002 Insert 1 of 3 Lab: Flanagan 329 |