Laboratory Safety Plan for Allergen Sensitization and Challenge

| # *Process | ## Allergen Sensitization and Challenge |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Biohazard Peptide Allergen, aerosol and solution |
| *Hazardous Equipment | Hypodermic needles, ultrasonic nebulizer |
| *Potential Hazards | Skin, mucous membrane, and pulmonary exposure |
| *Personal Protective Equipment | Safety glasses, latex gloves, scrubs, full lab. coat, closed toe shoes. When weighing allergens in powder form, the technician must where a N95 particulate mask. |
| *Engineering and Ventilation Controls | All aerosol exposures must be done in either a fume hood or biological safety cabinet. |
| Special Use Procedures | Respiratory protection and N95 masks must be fit tested annually by EH&S. |
| Special Handling and Storage Requirements | 1. Specimens will be placed in labeled, well-constructed containers during collection, handling, processing, storage, transport, and shipping to prevent leakage. 2. Single-use, disposable gloves will be worn at all times when handling samples. Water- resistant lab coats, masks, and protective eyewear will be worn if mucous-membrane contact with blood or potentially infectious materials is anticipated. Protective clothing will be removed before leaving the laboratory. 3. Hands will be washed with a disinfectant soap after handling all samples or immediately after hand contamination. 4. Biological safety cabinets or fume hoods vented directly through stacks on the building will be used due to aerosol generation. In general, the hoods in (room #) of the (----) Bldg. will be used for aerosol delivery. 6. Pipetting or suctioning by mouth is forbidden. 7. Respirators will be worn when working with allergens in powder form outside of a hood; for example, when weighing. 8. Usage of needles should be limited to situations where there is no alternative available. Needles should not be bent, sheared, replaced in sheath or guard, or removed from the syringe following use. Sharps and broken glassware will be placed in approved sharps containers. 9. Eating, drinking, smoking, applying cosmetics or lip balm, or handling contact lenses will be strictly prohibited in the work area. No food or drinks will be stored in areas where potentially infected materials are present. Whenever practical, allergens should be handled within a fume hood to minimize pulmonary exposure. |
| *Spill and Accident Procedures | Contaminated articles should be decontaminated with an EPA approved disinfectant or placed in a biohazard waste container for disposal. |
| *Waste Minimization Plan |  |
| #### *Hazardous Waste Disposal | All waste material will be discarded through the ECU hazardous waste management system. |
| #### Decontamination Procedures | Work surfaces will be decontaminated with an 10% Quatracide or bleach after completing work or in the event of a spill. Large spills will be appropriately wiped up before applying disinfectant to surface. |
| Animal Care Precautions |  |
| *Chemical Procurement |  |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)