## Laboratory Safety Plan for Clinical Hematology, Abnormal Hematology and Coagulation Labs

| ### Process | Normal and abnormal hematology and coagulation labs |
| --- | --- |
| # Hazardous Chemical/Chemical Class | Flammable liquids (alcohols, acetone, formaldehyde, xylene substitute), poisons (arsenic), glutaraldehyde (carcinogen) |
| Hazardous Equipment | Centrifuge |
| # Potential Hazards | Fire; inhalation of toxic vapors, absorption through skin, eyes and mucous membranes; acetone, formaldehyde and xylene substitute and irritants |
| Personal Protective Equipment | Full-buttoned front closing fluid resistant lab coat; safety glasses; nitrile gloves; closed toed toes; long pants |
| Engineering and Ventilation Controls | Formaldehyde, Drabkin’s reagent (arsenic) and glutaraldehyde must be handled under a chemical fume hood |
| Special Use Procedures | Not applicable |
| Special Handling and Storage Requirements | Avoid ignition sources such as hot plates, Bunsen burners or heat lamps in vicinity of flammable liquids; dispose of chemical waste in labeled hazardous waste containers; be sure centrifuges are in proper working order |
| Spill and Accident Procedures | Clean up spills using the cat litter found in room ____. Sweep up absorbed liquids with broom and dustpan – using mask and protective clothing as needed. Place collected waste in a sealed bad and call EHS for pick-up. |
| Waste Minimization Plan | Order only the amount of chemicals needed for each semester’s labs; make up only as much working reagent as needed for each lab |
| Hazardous Waste Disposal | All hazardous waste material stored in properly labeled waste bottles and discarded at the end of each semester through ECU hazardous waste management |
| Decontamination Procedure | Not applicable |
| Animal Care Precautions | Not applicable |
| Chemical Procurement | Inventories kept by laboratory manager; minimum amounts of needed chemicals kept in-house |