Laboratory Safety Plan for Acid Washing Glassware

| (Required fields are noted with a *. This document may be saved in Microsoft Word and changed using the table commands.) # *Process | ## Acid washing glassware with either 10% Hydrochloric Acid or 5% Sulfuric Acid |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | 10% Hydrochloric Acid or 5% Sulfuric Acid |
| *Hazardous Equipment | None |
| *Potential Hazards | Exposure: absorption through the eyes and transport of toxic substance dissolved in the chemicals into the body; inhalation of toxic vapor. |
| *Personal Protective Equipment | Safety goggles, nitrile gloves (resistant to chemicals), fully buttoned lab coat, closed toed shoes. |
| *Engineering and Ventilation Controls | N/A |
| Designated Use Area for Carcinogens, Reproductive Toxins or Acute Toxins | Not applicable. |
| Special Use Procedures | Wear gloves at all times. Maintain proper ventilation. |
| Special Handling and Storage Requirements | Gloves are disposed of after use. Wash hands after wearing gloves. Rinse any skin that comes in contact with chemical reagents with copious amounts of water. |
| *Spill and Accident Procedures | Clean up spills as they occur if personnel have the appropriate materials and training. All other spills should be reported to EH&S immediately for clean up. Materials needed: Absorbent materials (vermiculite, paper towels, etc.), broom and dust pan, plastic bags, sealable can to contain clean up materials, protective clothing, gloves, and safety goggles. For minor spills: open windows or ventilate area; cover liquid spills with absorbent materials being careful not to breathe any dust for the absorbent material, place material in a plastic bag, use soap and water and discard that material as stated above; gently brush particulate spills with a broom and dust pan, being careful not to inhale any dust from the chemical, place in a plastic bag, use soap and water and discard any of these cleaning materials with the cleaned up particulate material into a sealable can. Contact EH&S for removal of sealed container, DO NOT place this container in the regular waste container. Use a buffer such as soda ash or lime for acid/base spills. |
| *Waste Minimization Plan | Minimize the use of all hazardous chemicals. |
| #### *Hazardous Waste Disposal | All hazardous waste material will be discarded through the ECU Hazardous Waste Management System. |
| #### Decontamination Procedures | Not applicable. |
| Animal Care Precautions | Not applicable. |
| *Chemical Procurement |  |
| *Revision Date | Insert ____ of ____ Lab___________________________________________ |