Laboratory Safety Plan for __soil organic matter oxidation_________________

| # *Process | ## soil organic matter oxidation |
| --- | --- |
| *Hazardous Chemical/ Chemical Class | Strong Class 5.1 oxidizer – 30% hydrogen peroxide |
| *Hazardous Equipment | None |
| *Potential Hazards | Fire; Contact with other material may cause a fire. Eye contact may result in permanent eye damage. May cause central nervous system effects. Causes eye and skin irritation and possible burns. Corrosive. May cause severe respiratory tract irritation with possible burns. May cause severe digestive tract irritation with possible burns. Mutagen. Light sensitive. May be harmful if swallowed. May cause blood abnormalities. |
| *Personal Protective Equipment | Chemical splash goggles; neoprene gloves; full lab coat, closed-toe shoes |
| *Engineering and Ventilation Controls | All handling of peroxide must be done in the chemical fume hood |
| Designated Use Area for Carcinogens, Reproductive Toxins or Acute Toxins | NA |
| Special Use Procedures |  |
| Special Handling and Storage Requirements | Keep away from heat, sparks, and flame. Do not store near combustible materials. Store in a secondary container. Keep container closed when not in use. Store in a cool, dry, well-ventilated area away from incompatible substances such as alkalies, oxidizable materials, finely divided metals, alcohols, and permanganates. |
| *Spill and Accident Procedures | Clean up spills immediately. Use water spray to disperse the gas/vapor. Remove all sources of ignition. Absorb spill using an absorbent, non-combustible material such as earth, sand, or vermiculite. Do not use combustible materials such as saw dust. Flush spill area with water. Provide ventilation. Do not get water inside containers. Keep combustibles (wood, paper, oil, etc.,) away from spilled material. |
| *Waste Minimization Plan | Do not use more than 5 ml per sample |
| #### *Hazardous Waste Disposal | Decant excess peroxide and discard through the ECU hazardous waste management system. |
| #### Decontamination Procedures | NA |
| Animal Care Precautions | NA |
| *Chemical Procurement | Procure small quantities (500 ml) |
| *Revision Date | Insert __1__ of __1__ Lab__ |

(Required fields are noted with a *.  This document may be saved in Microsoft Word and changed using the table commands.)