Principal Investigator: Department: Laboratory Location (Building): Room Number(s)(include shared spaces): Laboratory Close-Out Date (Last day PI will be on-site):

The purpose of this checklist is to assist PI’s in safely removing hazardous substances from a laboratory and confirming that the area is free from contamination. It is the responsibility of the department to schedule a final close-out inspection with EH&S within 10 business days prior to moving or vacating lab space.

(Please initial below)

| # Chemicals | # Yes | # No | # N/A |
| --- | --- | --- | --- |
| Refrigerators/freezers, areas under sinks, fume hoods, cabinets and shelves, and bench tops have been checked for chemical storage (including shared spaces). |  |  |  |
| All chemicals, specimens and containers have been fully labeled and discarded or transferred to the new owner. |  |  |  |
| Refrigerators and freezers have been emptied, defrosted, and cleaned. |  |  |  |
| Are there chemicals that require disposal or transfer to RECY-CHEM? If yes, refer to the Chemical Hygiene Plan (CHP) for instructions and contact EH&S for pick-up. |  |  |  |
| Will any chemicals be transferred to another investigator? If yes, make sure chemicals are stored in the proper containers and labeled properly, added to the chemical inventory and transfer Safety Data Sheets (SDSs) with chemicals. |  |  |  |
| Storage areas are clean: chemical residues, drips and spills are decontaminated and cleaned. |  |  |  |
| Disposable liners/covers removed from work surfaces. |  |  |  |
| Bench tops washed with soap and water. |  |  |  |
| All debris removed from fume hood; all surfaces washed with soap and water. |  |  |  |
| Are there chemicals to be transferred to another location? If yes, refer to the CHP for transport information. |  |  |  |
| All keys, including keys to locked cabinets have been returned. |  |  |  |
| Controlled Substances | Yes | No | N/A |
| All storage areas free of controlled substances. |  |  |  |
| Are there controlled substances to be disposed? Contact NC Drug Diversion Officer or EH&S. |  |  |  |
| Will any controlled substances be transferred to another investigator? Please note: Permission to transfer ownership of any controlled substances must be obtained from US Drug Enforcement Agency. |  |  |  |
| Are there controlled substances to be transported to another location? Please note: The license must be updated before relocated. Abandonment of any controlled substance is in violation of the DEA permit under which it is held. |  |  |  |
| Arrangements have been made to keep all Controlled Substances records for at least 3 years. |  |  |  |
| ### Compressed Gas Cylinders | ### Yes | ### No | ### N/A |
| ### Cylinders are properly labeled, including if empty. |  |  |  |
| Cylinders are disconnected and caps are replaced. |  |  |  |
| Non-returnable lecture bottles have been submitted for hazardous waste pick-up. |  |  |  |
| Empty cylinders are returned to vendors. |  |  |  |
| Will any compressed gas cylinders be transferred to another investigator? (Health Sciences locations must notify the Materials Management: Medical Storeroom. All others notify the vendor.) |  |  |  |
| Have all standing gas orders been terminated? (Health Sciences locations must notify the Materials Management: Medical Storeroom. All others notify the vendor.) |  |  |  |
| Will any compressed gas cylinders be transported to another location? See the ECU Compressed Gas Policy https://oehs.ecu.edu/industrial-hygiene/compressed-gas/ |  |  |  |
| Radioactive Materials | Yes | No | N/A |
| Radioactive waste must be labeled with a radioactive waste tag. |  |  |  |
| All radioactive material properly removed from the laboratory. Call 744-DUMP for removal of transfer of radioactive material. Please note: No radioactive material may be transferred to another investigator unless approved by the Radiation Safety Officer. |  |  |  |
| Laboratory has been surveyed by Radiation Safety. Possible areas of contamination: refrigerators, freezers, centrifuges, water baths, hoods, sinks, floors, and waste containers. |  |  |  |
| Drain tags removed from drains. |  |  |  |
| Radiation warning signs and labels removed from doors, hoods, and benches. |  |  |  |
| Any laser or laser system (Class 3B or 4) must be properly removed from the laboratory. Radiation Safety must oversee the transfer of equipment to another investigator or the decommissioning of equipment for surplus. |  |  |  |
| Biological Materials | Yes | No | N/A |
| Work surfaces and storage areas are decontaminated. |  |  |  |
| Biosafety Cabinets are decontaminated. |  |  |  |
| All equipment is disinfected and decontaminated. |  |  |  |
| All potentially contaminated sharps are disposed of in an approved sharps container. |  |  |  |
| Unwanted biological agents/materials are disposed of in appropriate containers and autoclaved if necessary. |  |  |  |
| All waste is clearly labeled and Biological Safety has been contacted for disposal. |  |  |  |
| Are there biological materials that need to be transferred to another location? If yes, contact Biological Safety for transport information. |  |  |  |
| Are there biosafety cabinets that need to be transferred to another location? If yes, contact Biological Safety for guidance on decontamination and relocation. |  |  |  |
| All media, specimens, and supplies are properly labeled and transferred to another investigator or appropriately discarded. |  |  |  |
| Biohazard warning signs and labels removed from doors, hoods, equipment, and benches, as necessary. |  |  |  |
| ## Equipment | ## Yes | ## No | ## N/A |
| ## All equipment disinfected and decontaminated. |  |  |  |
| ## Is there equipment to be transferred to another investigator or surplus property? If yes, contact the University Property Office for information. (See Surplus Property Policy) |  |  |  |
| ## Is any equipment connected to permanent building systems being removed for transfer with the exiting investigator? If yes, contact Facilities Services. |  |  |  |
| ## Has all broken glass been placed in a rigid puncture-resistant container and sealed for disposal by Housekeeping? |  |  |  |
| # Records | # Yes | # No | # N/A |
| ## A copy of the last current lab inventory has been provided to the department chair. |  |  |  |
| ## All keys, including those to cabinets, refrigerators, freezers, chemical storage cabinets, etc. have been given to the department chair. |  |  |  |
| ## The door posting has been updated. |  |  |  |
| ## 1Card has been turned in to the supervisor and security clearances have been removed. |  |  |  |
| ## A final lab inspection has been requested through EH&S. (For use when faculty vacate a lab only. Lab managers should provide final inspection for graduate students & lab staff.) |  |  |  |

| Additional ECU Information Sources: |  |
| --- | --- |
| Chemical Hygiene Plan | Chemical Hygiene Plan |
| Laboratory Safety Page | Laboratory Safety |
| New Lab Set Up Page | New Lab Setup |
| Hazardous Waste Management Page | Hazardous Waste Management |
| Biological Safety Page | Biological Safety |
| Radiation Safety Page | Radiation Safety |

I have, to the best of my knowledge, complied with the requirements of the ECU Laboratory Close-Out Plan and am not aware of any other items or special circumstances that are not listed on this form. Principal Investigator: _____________________________________________ Date:___________________ Department Chair: ________________________________________________ Date:___________________ New Principal Investigator: _________________________________________ Date:___________________

Inspection result: (Signatures will be affixed when satisfactory inspection is obtained) Lab Safety Manager: ________________________________________ Complete Re-inspection Radiation Safety Officer: _____________________________________ Complete Re-inspection